SR 2x2 switch

CAUTION:   Thisdevice can be damaged by electrostatic discharge (ESD).  Please follow standardESD procedures for handling and installation.

Direct Drive operation

- Operating Instructions:

1. Connect GREEN pin to Ground.

2. Connect RED pin as follow:

| Optical Connection | Red Pin |
| --- | --- |
| 1 - 2 / 3 - 4 | -------- |
| 1 - 4 / 3 - 2 | 5V |

- Note: Typical Operating Current: 50 mA. Operating voltage is +5VDC 5%.

Operating Instructions:

Note: This device can be damaged by connecting it incorrectly.

1. Connect GREEN pin to Ground.

2. Connect RED pin to +5 VDC  5.

3. Connect YELLOW pin as follows:

TTL Drive operation

| Optical Connection | TTL Input Level (yellow pin) |
| --- | --- |
| 1 - 2 / 3 - 4 | LOW |
| 1 - 4 / 3 - 2 | HIGH |

TTL logic high:  (2.0  HIGH  5.25 V)

TTL logic low :  (0  LOW  0.8 V)

Typical Operating Current: 25 mA plus 50 mA when port 1 selected.